质量检测报告

QUALITY CERTIFICATE

编号No：FQC 2018/04/12

品名：过一硫酸氢钾复合盐(Oxone)

NAME： Potassium Monopersulfate Compound

批号：

Batch NOMBER： 2018/04/12

分子式：

FORMULA:： 2KHSO5 KHSO4 K2SO4

日期：

DATE： 2018/04/12

分析报告-ANALYSIS REPORT

序号

1

2

3

4

5

项目

活性氧Active oxygen，%

活性组分Active component(KHSO5)，%

水分Moisture content，%

堆积密度Bulk density，g/ml

1%水溶液1%Solution

PH

3%水溶液3% Solution

通过美国筛分标准20目

6

粒径（%）

（through USS#20 mesh）

规格

≥4.53

≥42.8

≤0.15

0.95-1.30

Appro.2.3

Appro.2.0

Min.98

结果

4.61

43.73

0.03

1.15

2.16

1.85

99.70

Particle size 通过美国筛分标准200目

（through USS#200 mesh）

过硫酸氢钾

POTASSIUM PEROXYMONOPERSULFATE

7

成分

Effective

Assay

硫酸钾 POTASSIUM SULFATE

硫酸氢钾 POTASSIUM BISULFATE

Max.5

≥43%

≥28%

≥27%

4

实际测量含量：43.73%

实际测量含量：28.1%

实际测量含量：27.3%

结论：合格（QUALIFIED）有效浓度98%（硫酸钾和过硫酸氢钾有效含氧量之和）

COMMENT：

WE HEREBY CERTIFY THAT THE ABOVE MENTIONED PRODUCTS BEEN MADE SATISFACTORY IN ACCORDANCE WITH THE

REQUIREMENTS SPECIFIED IN THE CONTRACT AND THE RESULTS DESRIBED ABOVE ARE TRUE AND CORRECT IN EVERY DETAIL.

Signed by：

备注

Remark

1、必要进进行不确定值估算 IT MAY BE AN ESTIMATE VALUE IF NECCESSARY

润东源环保科技限公司

官网：www.china-rdy.com           电话：400-681-7955         邮箱：hg@china-rdy.com