DC-Sputtering

Power Supply

MARIS

User Manual and

Technical Data

Series GSW

MARIS DC-Sputtering Power Supplies

The MARIS-technology is designed for industrial production systems. Reliability and easy handling are characteristics of the power supplies. The fully automatic arc-handling guarantees best coating results and prevents operating errors. There are different interfaces modules available, so that a Maris can be integrated in any system.

Series GSW	Version 11-2006	2

MARIS DC-Sputtering Power Supplies

Contents

| 1. | Technical data | 5 |
| --- | --- | --- |
| 1.1. | Output power operating range | 8 |
| 2. | Connecting the unit | 13 |
| 2.1. | Scope of delivery | 13 |
| 2.2. | Installation of the unit | 13 |
| 2.3. | Water connection | 15 |
| 2.4. | Output voltage connection X1 | 16 |
| 2.5. | Mains connection X2 | 18 |
| 2.6. | Control socket X3 | 19 |
| 2.6.1. | Interface AS 4 | 19 |
| 2.6.2. | Interface AS 4 F | 23 |
| 2.6.3. | Profibus | 27 |
| 3. | Manual operation | 28 |
| 3.1. | Indications | 28 |
| 3.2. | Wiring the control socket X3 for manual operation | 30 |
| 3.2.1. | Interface AS 4 | 30 |
| 3.2.2. | Interface AS 4 F | 31 |
| 3.3. | Starting the unit | 31 |
| 3.4. | Options | 33 |
| 3.4.1. | Ignition voltage | 33 |
| 3.4.2. | Run-in program for new targets | 34 |
| 3.4.3 | Second voltage range | 35 |
| 3.4.4. | Recorder output | 35 |
| 4. | Remote-controlled operation | 36 |
| 4.1. | Switching ON/OFF | 36 |
| 4.2. | External set value setting | 37 |
| 4.3. | Options | 38 |
| 4.3.1. | Ignition voltage | 38 |
| 4.3.2. | Second voltage range | 39 |
| 4.3.3. | Run-in program for new targets | 39 |
| 4.4. | Execution time table | 40 |
| 4.4.1. | Interface AS 4 | 40 |
| 4.4.2. | Interface AS 4 F | 41 |
| 4.5. | Wiring examples – control socket X3 (AS 4) | 41 |
| 4.5.1. | Use of internal voltage source | 41 |
| 4.5.2. | Internal control voltage, external set value setting | 42 |
| 4.5.3. | External control voltage | 42 |
| 4.5.4. | Control outputs | 42 |
| 5. | The Maris Arc-Handling | 44 |
| 6. | Principle of operation | 48 |

Series GSW	Version 11-2006	3

MARIS DC-Sputtering Power Supplies

| 7. | Safety, service and warranty | 49 |
| --- | --- | --- |
| 7.1. | Safety instructions | 49 |
| 7.1.1 | General information | 49 |
| 7.1.2. | Hazards | 49 |
| 7.1.3. | Working on the system | 50 |
| 7.2. | Warranty | 51 |
| 7.3. | Manufacturer address | 51 |
| 8. | Annex | 52 |

Series GSW	Version 11-2006	4

MARIS DC-Sputtering Power Supplies

1. Technical Data

| Table 1 | GSW 200 | GSW 250 | GSW 300 |  |  | GSW 200/ |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 500/400 |
| Nominal power | 20kW | 25kW | 30kW |  |  | 20kW |
| Output voltage | 800V | 800V | 800V |  |  | 500V |
| Output current | 40A | 50A | 60A |  |  | 70A |
| Control |  | Current, voltage or power controlled |  |  |  |  |
| Modes of operation | Manual via front panel or external via A/D interface |  |  |  |  |  |
|  |  | AS 4, AS 4 F or Profibus |  |  |  |  |
| Set point | 0...100% of nominal current, voltage or power |  |  |  |  |  |
| Output accuracy | +/- 1% of nominal value for current, voltage and power |  |  |  |  |  |
| Arc suppression | Quenching time: 6µs up to 3ms, automatically |  |  |  |  |  |
|  | Delay time: 6µs up to 10ms, automatically |  |  |  |  |  |
| Noise emission |  | LpA < 70db (A) |  |  |  |  |
| Mains connection |  | 3 x 400V, +/-10%, 50/60Hz, PE |  |  |  |  |
| Power consumption | 22kVA | 28kVA | 33kVA |  |  | 22kVA |
| Fusing | 3 x 35A | 3 x 50A | 3 x 63A |  |  | 3 x 35A |
| Cooling |  | Water, > 3 l/min, 2 to 8 Bar, < 40° C |  |  |  |  |
|  | Forced air, maximum ambient temperature 40° C |  |  |  |  |  |
| Water connection |  | Swagelok SS-QC6 |  |  |  |  |
| Cooling capability of water | 900W | 1100W | 1300W |  |  | 950W |
| Cooling capability of air | 100W | 110W | 150W |  |  | 150W |
| Size | 19“ slide-in, 3HU x 700mm deep (without plug) |  |  |  |  |  |
| Weight | 49kg | 51kg | 53kg |  |  | 49kg |
| Mains cable | 4 x 6mm² | 4 x 10mm² | 4 x 10mm² |  |  | 4 x 6mm² |
| Output cable | 3 x 10mm² | 3 x 10mm² | 3 x 16mm² |  |  | 3 x 16mm² |
| Mains connection (X2) |  | Harting Han k4/0 |  |  |  |  |
| DC-connection (X1) |  | Harting Han 2 MOD |  |  |  |  |

Series GSW	Version 11-2006	5