ZJC透平油专用滤油机

适用范围：

用于处理不合格的透平油，广泛适用于电力、化工、造纸、治金等行业的蒸汽轮机、燃气轮机、水轮机的透平油在线处理，保证机组调节，润湿系统正常工作，采用新型的聚集、分离技术，亦可对低黏度液压油和冷却油及需要脱除大量水分和实现精密过滤杂质的其他低粘度润滑油。

本机是利用物理手段，在不影响油液组份及性能的前提下快速脱除油液中所含大量的水分、杂质及气体(乙炔、氢气、甲烷等)等有害成份；保证机组调节、润滑系统正常工作，延长机组的维修周期。

产品特性：

- 采用聚结真空技术、分子吸收、真空干燥、自动破乳化等技术于一体，不使用任何添加剂破乳，彻底破乳化。

- 独特的三元一次脱水法，既能分离出油中的大量水分，也能脱除油中的微量水份，脱水破乳化效率高，使严重浑浊乳化的油也能变得清洗透明。

- 设备采用逐级加密过滤系统设置。滤器可进行反复清洗使用，耗材成本低，使用寿命长，高精度滤器配置能有效去除油液中的细微颗粒杂质。

- 采用强风与水冷技术相结合的介质冷凝装置，对抽出的高温高热混合气体进行冷凝除水，提高设备的有效输出功率，大大延长配件的使用寿命，使设备更经久耐用；

- 加热器采用低负荷设计，保证油液在加热器中能均匀加热，能有效避免引起油质的裂解和老化等危害；

- 先进的电气系统及自动红外线油位控制系统，自动控制不会发生跑油现象；

- 可与汽轮机、水轮机等设备实现无人值守的在线运行，低噪音，符合世界高标准环保要求；

- 设备可不停机自动排水，使设备操作使用更简单、人性化；

- 油液酸值如果超标，可选配BZ再生装置配搭使用，降低酸值与介损，油处理后酸值（mgKOH/g）≤0.03；也能对已经串汽、漏水、老化、含酸、多泡、乳化的油品做净化处理,使油品的各项质量指标满足《GB/T 7596-2008》运行中汽轮机油质量标准》的要求。可在线破乳、脱水、除酸、除杂，使严重浑浊乳化的透平油变得清澈透明。

ZJC系列透平油专用滤油机技术参数表

| 指标名称 |  | 单位 | ZJC-10 |  | ZJC-20 | ZJC-30 | ZJC-50 | ZJC-75 | ZJC-100 | ZJC-150 | ZJC-200 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 流 量 |  | L/H | 600 |  | 1200 | 1800 | 3000 | 4500 | 6000 | 9000 | 12000 |
| 工作真空度 |  | Mpa | -0.06 ～ -0.096 |  |  |  |  |  |  |  |  |
| 工作压力 |  | Mpa | ≤0.3 |  |  |  |  |  |  |  |  |
| 平均无故障工作时间 |  | H | ≥4000 |  |  |  |  |  |  |  |  |
| 连续工作时间 |  | H | ≥150 |  |  |  |  |  |  |  |  |
| 运行温度 |  | ℃ | 40 ～ 70 |  |  |  |  |  |  |  |  |
| 加热功率 |  | kW | 15 |  | 18 | 30 | 45 | 63 | 72 | 72 | 84 |
| 总 功 率 |  | kW | 17 |  | 20 | 33 | 49 | 69 | 79 | 81 | 97 |
| 进(出)口管径 |  | DN | 25 |  | 25 | 25 | 32 | 40 | 40 | 50 | 60 |
| 设备重量 |  | Kg | 500 |  | 530 | 570 | 780 | 800 | 960 | 1050 | 1550 |
| 外形 尺寸 | 长 | mm | 1300 |  | 1350 | 1450 | 1500 | 1500 | 1800 | 1800 | 1800 |
|  | 宽 | mm | 1000 |  | 1000 | 1100 | 1150 | 1000 | 1500 | 1650 | 1500 |
|  | 高 | mm | 1450 |  | 1450 | 1500 | 1650 | 1750 | 1800 | 1900 | 2000 |
| 处理后油指标 |  |  |  |  |  |  |  |  |  |  |  |
| 油中含水量 |  |  |  | 80ppm |  |  |  |  |  |  |  |
| 油中含气量 |  |  |  | ≤0.1% |  |  |  |  |  |  |  |
| 过滤精度 |  |  |  | ≤5µm（可根据用户要求配置） |  |  |  |  |  |  |  |
| 破乳化值 |  |  |  | ≤10 min |  |  |  |  |  |  |  |

ZJC Turbine oil vacuum purifier

Purpose:

It is widely suitable for online processing of steam turbine, the gas turbine, the hydraulic turbine's oil in electric power, chemical industry, papermaking, metallurgy and so on to guarantee the unit adjusting and the normal work of the lubrication system.

It used physical method remove the water, impurities and gas(acetylene, hydrogen, methane, etc), it will not change the oil components and performance.

Features:

- it completely breaking the emulsion by adopt vacuum technology, molecular absorption, vacuum drying, automatically break emulsification technology.

- Used unique ternary once dehydration method, not only remove large amounts of water from oil, but also remove trace moisture from oil, with high efficiency of dehydration and breaking the emulsion, it can let the serious turbid emulsion oil become transparent cleaning.

- n be washed and repeatedly use, it have long life and  need low cost. High-precision filter configuration can effectively remove fine particulate impurities from the oil.

- it using a combination of strong wind and water-cooled technology media condensing unit, condensing and except water from high temperature and high fever mixed gas, it improved the effective output power of the equipment, and extend the life of equipment.

- It have adopt low-load design in heater, ensured it can heat oil uniformly, can effectively prevent cracking and aging hazards of oil.

- Advanced electrical systems and automatic infrared oil control system.

- it can unattended running with the turbine or other equipment, low noise, it correspond the high standard of world environmental protection requirements.

- ZJC with the automatic drainage device can drain continuously, it is easy operation and hommization.

- If oil acid value is too high, you can choice BZ regeneration device to mach ZJC, reduce the acid value and dielectric loss, after process, acid value ≤0.03(mgKOH/g), let quality indicator meet the standard of "GB / T 7596-2008" the operation of turbine oil quality standards ". it can demulsification, dehydration, remove acid and impurities online, let the turbid emulsified turbine oil become clear and transparent.

ZJC系列透平油专用滤油机技术参数表

| 指标名称 Name |  | 单位 unit | ZJC-10 |  | ZJC-20 | ZJC-30 | ZJC-50 | ZJC-75 | ZJC-100 | ZJC-150 | ZJC-200 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 流 量 Capacity |  | L/H | 600 |  | 1200 | 1800 | 3000 | 4500 | 6000 | 9000 | 12000 |
| 工作真空度 Work vacuum |  | Mpa | -0.06 ～ -0.096 |  |  |  |  |  |  |  |  |
| 工作压力 Work pressure |  | Mpa | ≤0.3 |  |  |  |  |  |  |  |  |
| 平均无故障工作时间 Trouble free work time |  | H | ≥4000 |  |  |  |  |  |  |  |  |
| 连续工作时间 Continuous work time |  | H | ≥150 |  |  |  |  |  |  |  |  |
| 运行温度 Work temperature |  | ℃ | 40 ～ 70 |  |  |  |  |  |  |  |  |
| 加热功率 Heat power |  | kW | 15 |  | 18 | 30 | 45 | 63 | 72 | 72 | 84 |
| 总 功 率 Total temperature |  | kW | 17 |  | 20 | 33 | 49 | 69 | 79 | 81 | 97 |
| 进(出)口管径 Diameter of inlet&outlet |  | DN | 25 |  | 25 | 25 | 32 | 40 | 40 | 50 | 60 |
| 设备重量 weight |  | Kg | 500 |  | 530 | 570 | 780 | 800 | 960 | 1050 | 1550 |
| 外形 尺寸 Dimensions | 长L | mm | 1300 |  | 1350 | 1450 | 1500 | 1500 | 1800 | 1800 | 1800 |
|  | 宽W | mm | 1000 |  | 1000 | 1100 | 1150 | 1000 | 1500 | 1650 | 1500 |
|  | 高H | mm | 1450 |  | 1450 | 1500 | 1650 | 1750 | 1800 | 1900 | 2000 |
| 处理后油指标 Oil target after processing |  |  |  |  |  |  |  |  |  |  |  |
| 油中含水量 Waster content |  |  |  | 80ppm |  |  |  |  |  |  |  |
| 油中含气量 Gas content |  |  |  | ≤0.1% |  |  |  |  |  |  |  |
| 过滤精度 cleanness |  |  |  | ≤5µm（可根据用户要求配置 According customer's requirement） |  |  |  |  |  |  |  |
| 破乳化值 Demulsifying Value |  |  |  | ≤10 min |  |  |  |  |  |  |  |