| # 20KW/25KVA Cummins Diesel generator Technical Specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| OPEN-TYPE |  |  |  |  |  |
| 20 kw diesel generator set(25 kva diesel generator) are drove by Cummins Brand diesel engines manufactured by DCEC,and are connected with brushless generator fo Marathon or Stamford series and switch-broad,according to the requirements of our clients.The gensets have special devices for water temperature,oil temperature,low oil pressure and over-speed protection,so the products have many characteristics such as high precise voltage adjustment,excellent dynamic performance,stable voltage,long time efficient and economicetc |  |  |  |  |  |
| Model |  |  | 20KW/25KVA |  |  |
| GENERATOR SET | Rated Frequency (Hz) |  | 50 |  |  |
|  | Rated Output Power (KW) |  | 20 |  |  |
|  | Rated Voltage (AC) (V) |  | 230/400 |  |  |
|  | Rated Current (AC) (A) |  | 36 |  |  |
|  | Power Factor (COSΦ) |  | 0.8 (lag) |  |  |
|  | Power Supply |  | AC, 3-phase, 4-lines |  |  |
|  | Diesel generator set size |  | 1625×745×1225 |  |  |
|  | Diesel generator Weight(kg) |  | 670 |  |  |
| ENGINE | Engine Brand |  | Cummins |  |  |
|  | Engine Model |  | 4B3.9-G1 |  |  |
|  | Number Of Cylinders |  | 4 |  |  |
|  | Engine build |  | Line |  |  |
|  | Fuel type |  | #0 Diesel (Natural temperature) |  |  |
|  | Fuel Consumption (g/KW/h) |  | ≤206 |  |  |
|  | Piston diameter and stroke |  | 102*120 |  |  |
|  | Rated Rotation Speed (r/min) |  | 1500 |  |  |
|  | Starting Method |  | 24V Electric starting |  |  |
|  | Cooling Method |  | Closed cycle water-cooled |  |  |
|  | Speed Regulation |  | E |  |  |
| ALTERNATOR | Alternator Brand |  | Marathon |  |  |
|  | Alternator Model |  | GM-20-4 |  |  |
|  | Rated Output Power (KW) |  | 20 |  |  |
|  | Phase & Threads |  | three phase four threads |  |  |
|  | Insulation Class |  | self excitation |  |  |
|  |  |  | Brushless |  |  |
|  |  |  | H |  |  |
|  | Voltage Regulation |  | AVR |  |  |
|  | Protection class |  | IP23 |  |  |
| Main Electric Performance |  |  |  |  |  |
| Diesel Generators Voltage |  |  |  |  |  |
| Stable State Rate |  | Instantaneous State Rate |  | Stable Time | Fluctuation Rate |
| 1% |  | -15%~+20% |  | 1S | 0.5% |
| Diesel Generator Frequency |  |  |  |  |  |
| Stable State Rate |  | Instantaneous State Rate |  | Stable Time | Fluctuation Rate |
| 3~4% |  | 8~12% |  | 5~10S | 0.5% |